Yorick Documentation

for functions, variables, and structures

defined in file hex.i

Printed: Mon Oct 20 12:01:15 2025

Contents:

HX_blkbnd

HX_block

bi_dir

c_adjust

conv3_rays

cs_adjust

hex24b_track

hex24f_track

hex5_track

hex_mesh

hex_mesh2

hex_query

hex_startflag

hex_triang

hydra_mesh

hydra_start

make_sphere

pic3_rays

reg_track

track_combine

track_integ

track_reduce

track_solve

FROM HX_blkbnd TO c_adjust

HX_blkbnd

struct HX_blkbnd {  /* must match hex.h */

long block;

long cell;

int orient;

}

HX_block

struct HX_block {   /* must match hex.h */

long stride(3);

long length(3);

long first;

long final;

}

bi_dir

/* DOCUMENT nlist = bi_dir(tracker, mesh, rays, slimits, c, s)

Perform hexX_track and track_reduce on a ray that enters

the problem at the given point on the ray.  This requires

tracking the ray in both directions from the given point,

hence this function name indicating bi-directional tracking.

This is unnecessary when the entry point search was over

the problem boundary, or when the SLIMITS for the rays

always lie in one direction relative to the starting point.

TRACKER is the function used to track the rays, normally

one of hex5_track, hex_24f_track, or hex24b_track.

MESH is the problem mesh returned by hex_mesh or hydra_mesh;

it should be generated using the entry option that finds

the cell containing the given point on the ray.

RAYS is the 3-by-nrays-by-2 array of rays, as for hex5_track

SLIMITS is nil or the ray tracking limits as for track_reduce

C, S, together with NLIST are the output arrays, as for

track_reduce

SEE ALSO: track_reduce, hex5_track, hex24f_track, hex24b_track,

track_combine

*/

c_adjust

/* DOCUMENT c_adjust, c, mesh

or c_adjust, c, mesh, 1

or c= c_adjust(c, mesh, how)

adjust the cell number array C returned by track_reduce to

allow for a different layout of cell arrays than the one assumed

by the tracking routines.  Two HOW values are currently

supported: 0 (or nil) if the cell arrays are the same shape as

the nodal arrays, but the non-existent cell is at the end of

each row rather than at the beginning.  And 1 if the cell arrays

are smaller by one along each dimension than the nodal arrays.

If you call c_adjust as a subroutine, the input C array

FROM c_adjust TO hex5_track

is modified; if you call it as a function, the input C is

unchanged and the new values returned.

SEE ALSO: track_reduce, hex5_track, cs_adjust

*/

conv3_rays

/* DOCUMENT conv3_rays(rays)

convert [p,q] representation to or from best_rays representation.

If the first dimension of RAYS is 3, returns 5-by-raydims array

of best_rays; if first dimension of RAYS is 5, returns 3-by-raydims-

by-2 [p,q] for use with hex5_track.

SEE ALSO: hex5_track, pic3_rays, best_rays

*/

cs_adjust

/* DOCUMENT nlist= cs_adjust(nlist, c, s, ireg)

adjust NLIST, C, S returned from track_reduce to remove transits

of cells for which IREG == 0.  Can be called before or after

c_adjust, depending on layout of IREG.

SEE ALSO: c_adjust

*/

hex24b_track

/* SEE hex5_track     */

hex24f_track

/* SEE hex5_track     */

hex5_track

/* DOCUMENT c= hex5_track(mesh, rays, s)

c= hex24f_track(mesh, rays, s)

c= hex24b_track(mesh, rays, s)

track 3 x Nrays x 2 RAYS through the 3D MESH.  RAYS(,,1) are

points on the rays, while RAYS(,,2) are normalized ray directions.

The c return value and the S parameter are a long and double

array respectively, with number of elements equal to the total

number of intersections of all the RAYS with faces of the MESH,

plus one for any RAY which misses MESH entirely.  The values of

c are:

[#hits,cell1,cell2,cell3,..., #hits,cell1,cell2,cell3,..., ...]

where each #hits is followed by the list of cell indices (assuming

i=1, j=1, and k=1 are present but meaningless in cell arrays --

that is, assuming zone centered arrays have the same dimensions

as XYZ rather than one less in each direction).  Rays which miss

the mesh entirely have #hits=1, all others have #hits>=2 since they

must exit.  #hits<0 means a ray reentered the mesh for abs(#hits)

more face crossings, but this currently cannot happen.  The values

of S correspond to c:

FROM hex5_track TO hex_mesh2

[s0,s1,s2,s3,..., s0,s1,s2,s3,..., ...]

which are the distances along the ray measured from RAYS(,,1) in

the direction of RAYS(,,2) where the ray pierces a cell face.  For

rays which miss the mesh, the value of s0 is a diagnostic telling

why they missed (see compiled code).

Function hex5_track uses the 5-tet decomposition for hexes,

which is not unique when the quad faces are non-planar.  You may

be able to get an idea of this effect by setting hex_triang the

opposite way and redoing the trace.

Functions hex24f_track and hex24b_track use the face and body

centered 24-tet decompositions for hexes.  These are unique;

however, hex_triang may in rare cases change the trace slightly,

since the entry search algorithm still involves triangulating

the surface quads.

SEE ALSO: hydra_mesh, hex_triang, reg_track, track_reduce, c_adjust,

pic3_rays, conv3_rays

*/

hex_mesh

/* DOCUMENT mesh= hex_mesh(xyz, bound, nbnds, &mbnds, nblk, &blks, start)

create a 3D mesh object from the multiblock mesh parameters

XYZ   is NBLK 3 x Ni x Nj x Nk coordinate arrays packed together

BOUND is NBLK 3 x Ni x Nj x Nk face boundary markers packed

NBNDS is length of MBNDS

MBNDS is HX_blkbnd describing each internal block boundary face

NBLK  is number of blocks

BLKS  is NBLK HX_block objects describing the block structure

START is 0-origin 6*cell+face index of first boundary face/cell

or -1-cell to trace from centroid of that cell to point

p on ray to begin tracking

SEE ALSO: hex5_track, hydra_mesh, hex_startflag

*/

hex_mesh2

/* DOCUMENT mesh= hex_mesh2(xyz, bounds)

old interface for hex_mesh

create a 3D mesh object from the 3 x Ni x Nj x Nk coordinate

array XYZ and the list of 6 BOUNDS:

BOUNDS(1), BOUNDS(2)  for the i=1,Ni boundaries

BOUNDS(3), BOUNDS(4)  for the j=1,Nj boundaries

BOUNDS(5), BOUNDS(6)  for the k=1,Nk boundaries

The BOUNDS values are:

1   if this is a problem boundary

2   if this is a reflecting boundary

3   if this is a periodic boundary

SEE ALSO: hydra_mesh

*/

FROM hex_query TO hydra_mesh

hex_query

/* DOCUMENT start= hex_query(mesh, xyz, bound, mbnds, blks)

query a mesh created by hex_mesh, returning the arrays

passed to that function (these are not copies -- be careful

not to clobber them)

function return value is the start index

SEE ALSO: hex5_track, hydra_mesh

*/

hex_startflag

/* DOCUMENT old_flag= hex_startflag(new_flag)

possibly set flag to NEW_FLAG, always return OLD_FLAG, where

flag value is 0 (default) to begin search for new entry point

at previous entry point, 1 to begin search for new entry point

from mesh start face for every ray.  Any other value of NEW_FLAG

returns OLD_FLAG without changing it.

SEE ALSO: hex_mesh

*/

hex_triang

/* DOCUMENT old_flag= hex_triang(new_flag)

possibly set flag to NEW_FLAG, always return OLD_FLAG, where

flag value is 0 for default mesh triangulation, 1 for opposite

triangulation, and 2 on input to signal not to change the

current value.  The triangulation value can affect the result

of hex5_track if the quad faces of the mesh are not planar.

SEE ALSO: hex5_track

*/

hydra_mesh

/* DOCUMENT mesh= hydra_mesh(f)

or mesh= hydra_mesh(f, ublk, i0, j0, k0, face)

or mesh= hydra_mesh(f, ublk, i0, j0, k0)

read a 3D mesh object from the hydra PDB/Silo file F.

Note that the boundary arrays are adjusted to the hex convention

that cells with i=1, j=1, k=1 are missing, rather than the hydra

convention that i=imax, j=jmax, k=kmax are missing.

In the first form, the ray entry search will start on the

first open boundary face in the mesh.  If the actual problem

boundary is not convex, you need to identify a surface of

constant i, j, or k in the problem which is convex, and which

all the rays you intend to trace intersect.

UBLK is the user block number (starting from 0),

I0, J0, K0 are the (1-origin) logical coordinates of a

hydra *cell*.  Note that unlike hex cells, the hydra

cell bounded by nodes (1,1,1) and (2,2,2) is numbered (1,1,1).

FROM hydra_mesh TO pic3_rays

(Hex numbers it (2,2,2).)

FACE is the face number on cell (I0,J0,K0) which you want a

ray to enter.  0 means the -I face, 1 the +I face, 2 the -J

face, 3 the +J face, 4 the -K face, and 5 the +K face.

As you step from this cell to its neighbors, then to their

neighbors, and so on, this face must trace out a convex

surface for the ray entry search.  Rays not intersecting

this surface will not enter the problem; the ray trace

will begin at this surface, not at -infinity.

If FACE==-1 or is omitted (as in the third form), then the

given points on the rays are assumed to lie inside the mesh,

and a pseudo ray from the centroid of cell (I0, J0, K0) will be

tracked to the given point on each ray; the ray will be launched

into the cell containing that point.

SEE ALSO: hex_query, hex5_track, h_data, h_openb

*/

hydra_start

/* DOCUMENT hydra_start, mesh, start

change the starting cell of the hydra MESH (returned by hydra_mesh)

to START.  If called as a function, returns old start value.

SEE ALSO: hydra_mesh, h_data

*/

make_sphere

/* DOCUMENT make_sphere(radius, [imax,jmax,kmax],

[phi1, phi2], [theta1, theta2])

return a mesh (see hex_mesh) representing the given section

of the sphere of given RADIUS.  IMAX, JMAX, and KMAX are the

number of nodes (cells+1) in the radial, longitude (phi), and

colatitude (theta) directions, respectively.  Note that for

a right handed coordinate system, phi1<phi2 but theta1>theta2.

SEE ALSO: hex_mesh

*/

pic3_rays

/* DOCUMENT rays= pic3_rays(xpict, ypict, ray)

or rays= pic3_rays(xpict, ypict, ray, q_up)

Like picture_rays, but returns rays in the [p,q] representation

appropriate for hex5_track.

(XPICT,YPICT) are 2D arrays of pixel corners in the image plane;

RAY is the central ray (0,0) in (XPICT,YPICT) coordinates, given

in [p,q] representation (i.e. RAY is a 3-by-2 array).  The

optional Q_UP is a 3-vector specifying the orientation of the

y-axis in the picture plane (see theta_up, phi_up in picture_rays

FROM pic3_rays TO track_integ

for a description of default orientation).  Q_UP must not be

parallel to RAY(,2).

SEE ALSO: hex5_track, conv3_rays, picture_rays

*/

reg_track

/* DOCUMENT c= reg_track(x, y, z, rays, s)

track RAYS through regular mesh defined by the 1D coordinate

arrays X, Y, and Z.  Return values S and C are as for

hex5_track, where the mesh is numberof(X) by numberof(Y) by

numberof(Z).

SEE ALSO: hex5_track, track_reduce

*/

track_combine

/* DOCUMENT nlist = track_combine(nm,cm,sm, np,cp,sp, c, s)

combine two track_reduce results NM,CM,SM, and NP,CP,SP,

which represent the first and second halves of a set of

rays.  See bi_dir for a typical application.  The returned

NLIST is NM+NP, or NM+NP-1 for those rays where the

final CM is identical to the initial CP.

C, S, together with NLIST are the output arrays, as for

track_reduce.

SEE ALSO: track_reduce, bi_dir

*/

track_integ

/* DOCUMENT result= track_integ(nlist, transp, selfem, last)

integrates a transport equation by doing the sums:

transparency(i) = transparency(i-1) * TRANSP(i)

emissivity(i) = emissivity(i-1) * TRANSP(i) + SELFEM(i)

returning only the final values transparency(n) and emissivity(n).

The NLIST is a list of n values, so that many transport integrals

can be performed simultaneously; sum(NLIST) = numberof(TRANSP) =

numberof(SELFEM).  The result is 2-by-dimsof(NLIST).

If TRANSP is nil, result is dimsof(NLIST) sums of SELFEM.

If SELFEM is nil, result is dimsof(NLIST) products of TRANSP.

TRANSP and SELFEM may by 2D to do multigroup integrations

simultaneously.  By default, the group dimension is first, but

if LAST is non-nil and non-zero, the group dimension is second.

In either case, the result will be ngroup-by-2-by-dimsof(NLIST).

FROM track_integ TO track_solve

track_solve is the higher-level interface.

SEE ALSO: track_reduce, track_solve, track_solve

*/

track_reduce

/* DOCUMENT nlist= track_reduce(c, s)

or nlist= track_reduce(c, s, rays, slimits)

compresses the C and S returns from the tracking routines (see

hex5_track) to the following form:

[cell1,cell2,cell3,..., cell1,cell2,cell3,..., ...]

[s1-s0,s2-s1,s3-s2,..., s1-s0,s2-s1,s3-s2,..., ...]

returning nlist as

[#hits, #hits, ...]

In this form, any negative #hits are combined with the preceding

positive values, and #hits=1 (indicating a miss) appear as #hits=0

in nlist.  Hence, nlist always has exactly Nrays elements.

If RAYS is supplied, it is used to force the dimensions of the

returned nlist to match the dimensions of RAYS (the value of RAYS

is never used).  The RAYS argument need not have the trailing 2

dimension, so if you specified RAYS as [P,Q] if the call to

hex5_track, you can use just P or Q as the RAYS argument to

track_reduce.

If SLIMITS is supplied, it should be [smin,smax] or [smin,smax]-

by-dimsof(nlist) in order to reject input S values outside the

specified limits.  The C list will be culled appropriately, and

the first and last returned ds values adjusted.

With a non-zero flip= keyword, the order of the elements of

C and S within each group of #hits is reversed, so that a

subsequent track_solve will track the ray backwards.  If you

use this, both the ray direction input to the tracking routine

and any SLIMITS argument here should refer to the reverse of

the ray you intend to track.

SEE ALSO: hex5_track, c_adjust, track_solve, track_integ,

bi_dir, track_combine

*/

track_solve

/* DOCUMENT result= track_solve(nlist, c, s, akap, ekap, last)

integrates a transport equation for NLIST, C, and S returned

by track_reduce (and optionally c_adjust).  The RAYS argument

is used only to set the dimensions of the result.  AKAP and

EKAP are mesh-sized arrays of opacity and emissivity, respectively.

They may have an additional group dimension, as well.  The

units of AKAP are 1/length (where length is the unit of S),

while EKAP is (spectral) power per unit area (length^2), where

the power is what ever units you want the result in.  The

FROM track_solve TO track_solve

emission per unit volume of material is EKAP*AKAP; an optically

thick block of material emits EKAP per unit surface.

The NLIST is a list of n values, so that many transport integrals

can be performed simultaneously; sum(NLIST) = numberof(AKAP) =

numberof(EKAP).  The result is 2-by-dimsof(NLIST), where the

first element of the first index is the transmission fraction

through the entire ray path, and the second element of the

result is the self-emission along the ray, which has the same

units as EKAP.

If EKAP is nil, result is dimsof(NLIST) -- exactly the same as

the transparency (1st element of result) when both EKAP and AKAP

are specified.

If AKAP is nil, result is dimsof(NLIST).  In this case, EKAP

must have units of emission per unit volume instead of per unit

area; the result will be the sum of EKAP*S along each ray.

AKAP and EKAP may by 2D to do multigroup integrations

simultaneously.  By default, the group dimension is first, but

if LAST is non-nil and non-zero, the group dimension is last.

In either case, the result will be ngroup-by-2-by-dimsof(NLIST).

To use in conjuction with hex5_track, one might do this:

c= hex5_track(mesh, rays, s);

nlist= track_reduce(c, s, rays);

c_adjust, c, mesh;  // if necessary

result= track_solve(nlist, c, s, akap, ekap);

SEE ALSO: track_reduce, hex5_track

*/